# Landing Gear Code – Symbol & Code Name Reference

CADEM0016 GDP  |  LandingGear_ClassI_II_Sizing.m  |  University of Bristol, March 2026

All 80 variables extracted from code.  Code names follow MDO adp.* / loc.* / G.* convention (matching MDO screenshot).

| Symbol | Code Name | Parameter Definition | Default Value / Unit | Code Section | Implemented In (MATLAB line / block) |
| --- | --- | --- | --- | --- | --- |
| SECTION 1 – INPUTS: MASS PARAMETERS (adp.* equivalents from MDO screenshot) |  |  |  |  |  |
| MTOM | adp.MTOM | Maximum Take-Off Mass | 497870 kg | Inputs – Mass | Section 1 user inputs block |
| MLM_ratio | adp.MLM_ratio | MLM / MTOM ratio (widebody 0.85–0.95) | 0.90 [-] | Inputs – Mass | Section 1 – used to derive MLM = MLM_ratio × MTOM |
| OEM | adp.OEM | Operational Empty Mass (initial est.) | 145000 kg | Inputs – Mass | Section 1 (declared; not used in mass eqs. directly) |
| MLM | loc.MLM | Maximum Landing Mass (derived) | MLM_ratio × MTOM kg | Inputs – Mass | Derived positions block: MLM = MLM_ratio * MTOM |
| SECTION 1 – INPUTS: GEOMETRY PARAMETERS (G.* / loc.* equivalents) |  |  |  |  |  |
| l_fus | G.l_fus | Total fuselage length | 76.0 m | Inputs – Geometry | Section 1 user inputs; used in all position calcs (x = frac × l_fus) |
| R_fus | G.R_fus | Fuselage outer radius (gear-length floor) | 3.15 m | Inputs – Geometry | Section 1; used in h_gear_min_engine = R_fus + h_engine_clearance |
| x_tail_frac | loc.x_tail_frac | Fuselage fraction – tail contact point | 0.98 [-] | Inputs – Geometry | Section 1; x_tail = x_tail_frac × l_fus → tail-strike calc (Step 5) |
| h_tail_contact | loc.h_tail_contact | Height of tail contact above gear pivot | 1.50 m | Inputs – Geometry | Section 1; h_tail_gnd = h_tail_contact + h_gear_min_engine (Step 5) |
| x_nlg_frac | loc.x_nlg_frac | NLG position as fraction of fuselage | 0.05 [-] | Inputs – Geometry | Section 1; x_nlg = x_nlg_frac × l_fus (Step 9) |
| n_legs | loc.n_legs | Number of main landing gear legs | 4 [-] | Inputs – Geometry | Section 1; used in F_static_leg = MLM×g×0.90 / n_legs |
| SECTION 1 – INPUTS: CG ENVELOPE (Snorri §13.3.1 Steps 1–2, Fig.13-23) |  |  |  |  |  |
| x_cg_fwd_frac | loc.x_cg_fwd_frac | Most forward CG as fraction of fuselage | 0.43 [-] | Inputs – CG Envelope | Section 1; x_cg_fwd = x_cg_fwd_frac × l_fus → used in nose load (Step 9) |
| x_cg_aft_frac | loc.x_cg_aft_frac | Most aft CG as fraction of fuselage | 0.47 [-] | Inputs – CG Envelope | Section 1; x_cg_aft = x_cg_aft_frac × l_fus → tip-back β, aft nose load |
| x_cg_nom_frac | loc.x_cg_nom_frac | Nominal (design) CG fraction | 0.45 [-] | Inputs – CG Envelope | Section 1; x_cg_nom = x_cg_nom_frac × l_fus → overturn angle, nom load |
| SECTION 1 – INPUTS: ENGINE & MATERIAL (loc.* equivalents from MDO) |  |  |  |  |  |
| D_fan | loc.D_fan | Engine fan diameter | 3.20 m | Inputs – Engine/Mat. | Section 1; h_engine_clearance = 0.23 × D_fan (Step 4, Snorri §13.1.3) |
| sigma_y | loc.Al.sig_all | Yield strength – 300M steel (tube sizing) | 1000 MPa | Inputs – Engine/Mat. | Section 5 (II.5): compressive stress check σ_c ≤ σ_y / SF |
| E_steel | loc.E_steel | Young's modulus – structural steel | 200 GPa | Inputs – Engine/Mat. | Section 5 (II.5): Euler buckling I_req = SF×Fp×(K×L)²/(π²×E) |
| rho_steel | loc.CF.rho | Steel density (main fitting tube) | 7850 kg/m³ | Inputs – Engine/Mat. | Section 5 (II.5): M_tube_one = rho_steel × V_tube |
| g | SI.g | Gravitational acceleration (constant) | 9.81 m/s² | Inputs – Engine/Mat. | Used throughout: F = m×g, Breguet, stroke eq. |
| SECTION 2 – DERIVED POSITIONS (absolute metres from nose) |  |  |  |  |  |
| x_cg_fwd | loc.x_cg_fwd | Forward CG limit (absolute) | x_cg_fwd_frac × l_fus m | Derived – Positions | Derived block: x_cg_fwd = x_cg_fwd_frac * l_fus |
| x_cg_aft | loc.x_cg_aft | Aft CG limit (absolute) | x_cg_aft_frac × l_fus m | Derived – Positions | Derived block: x_cg_aft = x_cg_aft_frac * l_fus |
| x_cg_nom | loc.x_cg_nom | Nominal CG (absolute) | x_cg_nom_frac × l_fus m | Derived – Positions | Derived block: x_cg_nom = x_cg_nom_frac * l_fus |
| x_nlg | loc.x_nlg | NLG contact point from nose (absolute) | x_nlg_frac × l_fus m | Derived – Positions | Derived block: x_nlg = x_nlg_frac * l_fus |
| x_tail | loc.x_tail | Tail contact point from nose (absolute) | x_tail_frac × l_fus m | Derived – Positions | Step 5: x_tail = x_tail_frac * l_fus |
| h_engine_clearance | loc.h_clr | Required engine-to-ground clearance | 0.23 × D_fan m | Derived – Positions | Step 4: h_engine_clearance = 0.23 * D_fan |
| h_gear_min_engine | loc.h_gear_min | Minimum compressed gear length | R_fus + h_clr m | Derived – Positions | Step 4: h_gear_min_engine = R_fus + h_engine_clearance |
| h_tail_gnd | loc.h_tail_gnd | Tail contact height above ground | h_tail_contact + h_gear_min m | Derived – Positions | Step 5: h_tail_gnd = h_tail_contact + h_gear_min_engine |
| theta_ts_target | loc.theta_ts_tgt | Target tail-strike angle | 11.0 deg | Derived – Positions | Step 5: used to back-calculate dist_mg_tail_req |
| dist_mg_tail_req | loc.dist_mg_tail | Required MLG→tail horiz. distance | h_tail_gnd / tan(θ_ts) m | Derived – Positions | Step 5: dist_mg_tail_req = h_tail_gnd / tand(theta_ts_target) |
| x_mg_from_ts | loc.x_mg_from_ts | MLG position implied by tail-strike | x_tail − dist_mg_tail m | Derived – Positions | Step 5: x_mg_from_ts = x_tail - dist_mg_tail_req |
| x_mg | loc.x_mg | MLG longitudinal position from nose | x_mg_from_ts m | Derived – Positions | Steps 6–8: x_mg = x_mg_from_ts (set after STEP 5) |
| theta_ts_actual | loc.theta_ts | Actual tail-strike angle (verified) | atan(h_tail_gnd/dist) deg | Derived – Positions | Step 8: theta_ts_actual = atand(h_tail_gnd / dist_mg_tail_actual) |
| wheelbase | loc.wheelbase | NLG to MLG distance (wheelbase) | x_mg − x_nlg m | Derived – Positions | Step 9: wheelbase = x_mg - x_nlg |
| dist_mg_cg_nom | loc.dist_mg_cg_nom | MLG to nominal CG (longitudinal) | x_mg − x_cg_nom m | Derived – Positions | Step 9: used in R_N_nom |
| dist_mg_cg_fwd | loc.dist_mg_cg_fwd | MLG to forward CG | x_mg − x_cg_fwd m | Derived – Positions | Step 9: used in R_N_fwd (max nose load) |
| dist_mg_cg_aft | loc.dist_mg_cg_aft | MLG to aft CG | x_mg − x_cg_aft m | Derived – Positions | Step 9: used in R_N_aft (min nose load) & tip-back β |
| R_N_nom | loc.R_N_nom | NLG reaction – nominal CG | MTOM×g×dist_nom/B N | Derived – Positions | Step 9: R_N_nom = MTOM * g * dist_mg_cg_nom / wheelbase |
| R_N_fwd | loc.R_N_fwd | NLG reaction – forward CG (max) | MTOM×g×dist_fwd/B N | Derived – Positions | Step 9: R_N_fwd = MTOM * g * dist_mg_cg_fwd / wheelbase |
| R_N_aft | loc.R_N_aft | NLG reaction – aft CG (min) | MTOM×g×dist_aft/B N | Derived – Positions | Step 9: R_N_aft = MTOM * g * dist_mg_cg_aft / wheelbase |
| nose_pct_nom | loc.nose_pct_nom | NLG load share % at nominal CG | R_N_nom/(MTOM×g) % | Derived – Positions | Step 9: nose_pct_nom = R_N_nom/(MTOM*g)*100 |
| nose_pct_fwd | loc.nose_pct_fwd | NLG load share % at forward CG | R_N_fwd/(MTOM×g) % | Derived – Positions | Step 9: displayed & checked vs 10–20% |
| nose_pct_aft | loc.nose_pct_aft | NLG load share % at aft CG | R_N_aft/(MTOM×g) % | Derived – Positions | Step 9: displayed & checked vs ≥5% |
| h_cg_approx | loc.h_cg | CG height above ground (approximation) | h_gear_min + 1.0 m | Derived – Positions | Used in tip-back β and overturn angle θ |
| beta_tipback | loc.beta | Tip-back angle β (Snorri Fig.13-24) | atan(dist_aft/h_cg) deg | Derived – Positions | Tip-back block: beta = atand(dist_mg_cg_aft / h_cg_approx) |
| main_pct_nom | loc.main_pct_nom | Main gear load share % at nominal CG | 100 − nose_pct_nom % | Derived – Positions | Check: main_pct_nom = 100 - nose_pct_nom; target 85–95% |
| B_wb | loc.B_wb | Wheelbase (alias for overturn calc) | wheelbase m | Derived – Positions | Steps 10–14: B_wb = wheelbase |
| D_cg | loc.D_cg | NLG to nominal CG (longitudinal) | x_cg_nom − x_nlg m | Derived – Positions | Steps 10–14: D_cg = x_cg_nom - x_nlg |
| E_cg | loc.E_cg | CG height for overturn (alias) | h_cg_approx m | Derived – Positions | Steps 10–14: E_cg = h_cg_approx |
| theta_ov | loc.theta_ov | Overturn angle θ (Snorri Fig.13-27) | atan(E/Y) deg | Derived – Positions | Steps 10–14 loop: theta_ov = atand(E_cg/Y_ov); limit < 63° |
| T_track | loc.T_track | Minimum MLG track width (lateral) | iterated m | Derived – Positions | Steps 10–14 loop: iterate T_track until theta_ov ≤ 63° |
| SECTION 3 – CLASS I: STATISTICAL MASS ESTIMATE (4% of MTOM, Raymer Table 15.2) |  |  |  |  |  |
| LG_mass_frac | loc.LG_mass_frac | LG mass fraction (statistical) | 0.04 [-] | Class I – Mass | Section 3: M_LG_total_CI = LG_mass_frac * MTOM |
| M_LG_total_CI | loc.M_LG_total | Class I total LG mass | LG_mass_frac × MTOM kg | Class I – Mass | Section 3: M_LG_total_CI = 0.04 * MTOM |
| M_LG_main_CI | loc.M_LG_main | Class I main gear mass (90% of total) | 0.90 × M_LG_total kg | Class I – Mass | Section 3: M_LG_main_CI = 0.90 * M_LG_total_CI |
| M_LG_nose_CI | loc.M_LG_nose | Class I nose gear mass (10% of total) | 0.10 × M_LG_total kg | Class I – Mass | Section 3: M_LG_nose_CI = 0.10 * M_LG_total_CI |
| SECTION 4 – CLASS II: TYRE SIZING & WHEEL COUNT (Raymer Table 11.1) |  |  |  |  |  |
| MTOW_kg_per_tyre | loc.W_per_tyre | Target load per tyre (flotation guide) | 25000 kg/tyre | Class II – Tyres | Section 4: guideline for n_MLG_wheels calculation |
| n_MLG_wheels | loc.n_MLG_wheels | Number of main landing gear wheels | ceil(0.9×MTOM/25000) [-] | Class II – Tyres | Section 4: n_MLG_wheels = ceil(0.9*MTOM/MTOW_kg_per_tyre); even |
| n_NLG_wheels | loc.n_NLG_wheels | Number of nose landing gear wheels | 2 [-] | Class II – Tyres | Section 4: fixed at 2 (standard dual nose gear) |
| W_wheel_lbs | loc.W_wheel_lbs | Load per main-gear tyre | (0.9×MTOM×2.2046)/n lbf | Class II – Tyres | Section 4: W_wheel_lbs = (0.9*MTOM*2.2046)/n_MLG_wheels |
| A_tyre | loc.A_tyre | Tyre diameter regression coefficient A | 0.89 [-] | Class II – Tyres | Section 4: Raymer Table 11.1 Type VII (A_d=0.89, B_d=0.361) |
| B_tyre | loc.B_tyre | Tyre diameter regression exponent B | 0.361 [-] | Class II – Tyres | Section 4: Raymer Table 11.1 Type VII |
| D_tyre_in | loc.D_tyre_in | Tyre diameter | A×W^B in | Class II – Tyres | Section 4: D_tyre_in = A_tyre * W_wheel_lbs^B_tyre |
| W_tyre_in | loc.W_tyre_in | Tyre width (≈ 0.35 × diameter) | 0.35×D_tyre_in in | Class II – Tyres | Section 4: W_tyre_in = 0.35 * D_tyre_in |
| D_tyre_m | loc.D_tyre_m | Tyre diameter (SI) | D_tyre_in × 0.0254 m | Class II – Tyres | Section 4: D_tyre_m = D_tyre_in * 0.0254 |
| R_tyre | loc.R_tyre | Tyre radius | D_tyre_m / 2 m | Class II – Tyres | Section 4: R_tyre = D_tyre_m / 2 |
| x_t | loc.x_t | Tyre deflection under load | 0.152 m (6 in) | Class II – Tyres | Section 4: x_t = 6 * 0.0254 (Hoole slide p.15) |
| SECTION 5 – CLASS II.5: SHOCK ABSORBER & TUBE SIZING (Hoole slides p.15–18, Schmidt p.865) |  |  |  |  |  |
| V_z | loc.V_z | Vertical descent rate at touchdown | 3.048 m/s (10 ft/s) | Class II.5 – Shock/Tube | Section 5: V_z = 10 * 0.3048 (FAR/CS-25 certification) |
| lambda_vec | loc.lambda_vec | Landing reaction factor sweep vector | 1.1 : 0.05 : 1.8 [-] | Class II.5 – Shock/Tube | Section 5 loop: lambda_vec = 1.1:0.05:1.8 |
| P_static | loc.P_static | Static gas pressure in oleo (1500 psi) | 10.34 MPa | Class II.5 – Shock/Tube | Section 5: P_static = 1500 * 6894.76 [Pa] |
| F_static_leg | loc.F_static_leg | Static force per MLG leg at MLM | MLM×g×0.9/n_legs N | Class II.5 – Shock/Tube | Section 5: F_static_leg = (MLM*g*0.90)/n_legs |
| xs_vec | loc.xs_vec | Stroke length array (vs λ) | m | Class II.5 – Shock/Tube | Section 5 loop: xs = (Vz²/(2λg) − 0.5×xt)/0.75 |
| Fpeak_vec | loc.Fpeak_vec | Peak force per leg array (vs λ) | N | Class II.5 – Shock/Tube | Section 5 loop: F_peak = lam * F_static_leg |
| M_tube_vec | loc.M_tube_vec | Leg mass array (vs λ) | kg | Class II.5 – Shock/Tube | Section 5 loop: M_leg = M_tube_one * 1.50 |
| lambda_design | loc.lambda_des | Design landing reaction factor | 1.2 [-] | Class II.5 – Shock/Tube | Section 5: lambda_design = 1.2 (large civil, Gudmundsson p.568) |
| xs_d | loc.xs_d | Stroke length at design λ | m | Class II.5 – Shock/Tube | Section 5: xs_d = xs_vec(idx_d) |
| Fp_d | loc.Fp_d | Peak force per leg at design λ | N | Class II.5 – Shock/Tube | Section 5: Fp_d = Fpeak_vec(idx_d) |
| Ml_d | loc.Ml_d | Leg mass at design λ | kg | Class II.5 – Shock/Tube | Section 5: Ml_d = M_tube_vec(idx_d) |
| A_sl_d | loc.A_sl_d | Piston cross-sectional area at design λ | F_static/P_static m² | Class II.5 – Shock/Tube | Section 5: A_sl_d = F_static_leg / P_static |
| d_sl_d | loc.d_sl_d | Slider (piston) diameter at design λ | 2√(A_sl/π) m | Class II.5 – Shock/Tube | Section 5: d_sl_d = 2 * sqrt(A_sl_d / pi) |
| SECTION 5 – CLASS II.5: PHYSICS-BASED MASS TOTALS |  |  |  |  |  |
| M_main_phys | loc.M_main_phys | Physics-based total main gear mass | n_legs × Ml_d kg | Class II.5 – Mass | Section 5: M_main_phys = n_legs * Ml_d |
| M_nose_phys | loc.M_nose_phys | Physics-based nose gear mass | (0.10/0.90)×M_main kg | Class II.5 – Mass | Section 5: M_nose_phys = (0.10/0.90) * M_main_phys |
| M_total_phys | loc.M_total_phys | Physics-based total LG mass | M_main + M_nose kg | Class II.5 – Mass | Section 5: M_total_phys = M_main_phys + M_nose_phys |
| RAYMER λ-CORRECTED CROSS-CHECK (Schmidt p.865, Raymer Ch.15) |  |  |  |  |  |
| L_gear_in | loc.L_gear_in | Total MLG length (compressed + stroke) | (h_gear_min + xs_d)/0.0254 in | Raymer Cross-check | Section 5 Raymer block: L_gear_in = (h_gear_min_engine + xs_d)/0.0254 |
| L_nlg_in | loc.L_nlg_in | NLG length estimate | 0.80 × L_gear_in in | Raymer Cross-check | Section 5 Raymer block: L_nlg_in = L_gear_in * 0.80 |
| W_land_lbs | loc.W_land_lbs | Maximum Landing Weight | MLM × 2.2046 lbf | Raymer Cross-check | Section 5 Raymer block: W_land_lbs = MLM * 2.2046 |
| W_MLG_R_lbs | loc.W_MLG_R | Raymer MLG mass (λ-corrected) | 0.0117×λ×W^0.95×L^0.43×N^0.525 lbf | Raymer Cross-check | Section 5 Raymer block – Schmidt p.865 regression |
| W_NLG_R_lbs | loc.W_NLG_R | Raymer NLG mass | 0.048×W^0.67×L^0.43×N^0.525 lbf | Raymer Cross-check | Section 5 Raymer block – Schmidt p.865 regression |
| M_MLG_R | loc.M_MLG_R | Raymer main gear mass (SI) | W_MLG_R_lbs / 2.2046 kg | Raymer Cross-check | Section 5: M_MLG_R = W_MLG_R_lbs / 2.2046 |
| M_NLG_R | loc.M_NLG_R | Raymer nose gear mass (SI) | W_NLG_R_lbs / 2.2046 kg | Raymer Cross-check | Section 5: M_NLG_R = W_NLG_R_lbs / 2.2046 |
| M_LG_R | loc.M_LG_R | Raymer total LG mass (SI) | M_MLG_R + M_NLG_R kg | Raymer Cross-check | Section 5: M_LG_R = M_MLG_R + M_NLG_R |

Section colour key:  ■ Inputs – Mass  ■ Inputs – Geometry  ■ Inputs – CG Envelope  ■ Inputs – Engine/Mat.  ■ Derived – Positions  ■ Class I – Mass  ■ Class II – Tyres  ■ Class II.5 – Geometry  ■ Class II.5 – Shock/Tube  ■ Class II.5 – Mass  ■ Raymer Cross-check