LANDING GEAR CLASS I / II / II.5 SIZING TOOL
 CADEM0016 – MSc Group Design Project
=======================================================

--- SECTION 2: CLASS I ESTIMATES ---

Class I: MLM = 448083 kg  (MLM/MTOM = 0.90)
Class I: Total LG mass  = 19915 kg (4.0% MTOM)
         Main LG mass   = 17923 kg
         Nose LG mass   = 1991 kg

Class I: Recommended main gear wheels = 18 (+ 2 nose)
         Mass per MLG tyre = 24.9 t

--- SECTION 3: CLASS II ESTIMATES ---

Class II: Tyre diameter = 1.16 m (45.7 in)
          Tyre width    = 0.41 m (16.0 in)
          Tyre radius   = 0.581 m

Class II: Static main gear total load = 4396 kN
          Static nose gear total load  = 488 kN
          Load per MLG leg             = 1099 kN

Class II: Longitudinal placement:
          NLG position  = 3.80 m from nose
          MLG position  = 39.52 m from nose
          CG position   = 34.20 m from nose
          Wheelbase     = 35.72 m
          NLG load (static) = 727 kN  (14.9% of MTOM)

Class II: Tail-strike angle = 8.5 deg  (target: 8–15 deg)
          OK: within safe range
          Tip-back angle β = 44.0 deg  (must be > 8.5 and < 25 deg)
          WARNING: Large elevator input required for rotation.

Class II: Lateral placement (turnover constraint):
          Required track width T = 16.00 m  (turnover angle = 69.7 deg)
          ICAO E max track = 14.0 m  |  ICAO F max track = 16.0 m
          INFO: ICAO F (taxi) required (track > 14 m)

Class II: Gear length from engine clearance = 3.886 m
          (engine clearance required = 0.736 m  = 0.23 x 3.20 m fan)

--- SECTION 4: CLASS II.5 PHYSICS-BASED ESTIMATES ---

4.1  Shock Absorber Stroke Length  (Class II.5)
     Energy equation: 0.5*m*Vz^2 = lambda*m*g*(0.75*xs + 0.5*xt)
     Vz = 3.048 m/s  |  xt = 0.1524 m  |  MLM = 448083 kg

     lambda |  xs [m]  | xs+xt [m] | F_peak [kN/leg] | M_leg_tube [kg]
     -------|----------|-----------|-----------------|----------------
      1.10  |   0.4724 |    0.6248 |          1208.8  |         21.4
      1.15  |   0.4474 |    0.5998 |          1263.8  |         20.3
      1.20  |   0.4245 |    0.5769 |          1318.7  |         19.2
      1.25  |   0.4035 |    0.5559 |          1373.7  |         18.3
      1.30  |   0.3841 |    0.5365 |          1428.6  |         17.4
      1.35  |   0.3661 |    0.5185 |          1483.5  |         16.6
      1.40  |   0.3494 |    0.5018 |          1538.5  |         15.8
      1.45  |   0.3338 |    0.4862 |          1593.4  |         15.1
      1.50  |   0.3193 |    0.4717 |          1648.4  |         14.5
      1.55  |   0.3057 |    0.4581 |          1703.3  |         13.8
      1.60  |   0.2930 |    0.4454 |          1758.3  |         13.3
      1.65  |   0.2810 |    0.4334 |          1813.2  |         13.2
      1.70  |   0.2698 |    0.4222 |          1868.2  |         13.1
      1.75  |   0.2592 |    0.4116 |          1923.1  |         12.9
      1.80  |   0.2491 |    0.4015 |          1978.1  |         12.8

  >> Design choice: lambda = 1.20
     Stroke length xs        = 0.4245 m (16.71 in)
     Peak force per MLG leg  = 1318.7 kN
     Slider (piston) diameter = 0.3489 m (13.74 in)
     Physics-based main gear mass = 77 kg  (19.2 per leg)
     Physics-based nose gear mass = 9 kg
     Physics-based TOTAL LG mass  = 85 kg  (0.02% MTOM)

  >> Raymer cross-check (λ-corrected, Schmidt p.865):
     Main gear mass = 130851 kg
     Nose gear mass = 2688 kg
     Total LG mass  = 133540 kg  (26.82% MTOM)

=======================================================
 RESULTS SUMMARY
=======================================================
 Method          | Total LG [kg] | Main [kg] | Nose [kg]
 ----------------|---------------|-----------|----------
 Class I  (4%)   |         19915 |     17923 |     1991
 Class II.5 tube |            85 |        77 |        9
 Raymer (λ=1.2) |        133540 |    130851 |     2688

 Geometry:
   NLG position          = 3.80 m from nose
   MLG position          = 39.52 m from nose
   Track width (min)     = 16.00 m
   Gear length (engine)  = 3.886 m
   Stroke length (λ=1.2) = 0.4245 m
   Tyre diameter         = 1.162 m  (45.7 in)
   Slider diameter       = 0.3489 m  (13.74 in)

 Geometric constraints:
   Tail-strike angle     = 8.5 deg  (limit: <15, aim 8-12)
   Tip-back angle β      = 44.0 deg  (must be >8.5 and <25)
   Turnover angle θ      = 69.7 deg  (limit: <63)
=======================================================