Class I Models (Statistical/Empirical Correlations – MDO Development Initial Estimates)
       1. Maximum Landing Weight (MLW) Estimation:

- Calculation: MLW = 0.9 × MTOM = 0.9 × 497,870 kg = 448,083 kg.

- Classification Justification: This class I assumption is based on historical ratios related to widebody freighters. Large transport aircraft typically have 0.85 – 0.95. So, we can take 0.9 for our assumption. Example B777 has 0.86, and Airbus A350 has 0.89 ratios.

- Engineering Justification: So, 0.9 is reasonable. Aircraft with large balloon tyres or many landing gear wheels to increase floatation. So, High MLW/MTOM relates to the (less fuel burn-off vs. pax aircraft)

2. Total Landing Gear Mass Estimate:

- Calculation: approximately 20,000 kg (implied ~4% MTOM); main 17,600 kg (90%), nose 2,400 kg (10%).

- Classification Justification: Class I statistical fraction (Raymer Table 15.2: gear 0.033-0.057 MTOW for transports, avg ~0.04;

- Engineering Justification: Aligns with widebody norms (e.g., A380 ~4.5% per Raymer p. 598); 90/10 split is empirical 7: 85-95% main load share. For our unconventional design, justify via reference (spec p. 12: compared to B777F gear mass ~4% of 348t MTOM ≈14,000 kg, scaled up for 498t).

3. Number of Wheels (Initial Selection):

- Calculation: 20 wheels (main 18, nose 2); compared to A380 (575t, 20 wheels), scaled for 500t.

- Classification Justification: Class I historical comparison (slide p. 6: explore MTOW vs. wheels relationship; Raymer p. 343: scale from similar aircraft like B747/A380 for flotation).

- Engineering Justification: Ensures certification (slide p. 6: 2 wheels/axle for braking with flat tire); multi-bogey for high MTOW (Raymer p. 340). (two 4-wheel wing, two 5-wheel body) fits track ~10 m (slide p. 9: ICAO E/F) and folding wingtips (spec p. 7: novel features).

Class II Models (Semi-Empirical with Simplified Physics – MDO Development Refined Estimates)

These combine correlations with geometry/loads (handbook p. 3: "simplified physics"; Raymer Ch. 11/15: equations with terms like wheels, length, load factor).

- Tire Sizing and Wheel Load:

  - Calculation: Wheel load = 90% MTOM lbs / 18 = 54,876 lbs; D = 0.89 × (54,876) ^0.361 ≈1.16 m; W ≈0.41 m (Type VII).

  - Classification Justification: Class II semi-empirical (Raymer Table 11.1: power-law fits to catalog data with load; slide p. 6: tire selection for flotation, implying empirical scaling).

  - Engineering Justification: Ensures ~200 psi pressure (Raymer Table 11.3) for paved runways; justifies wheel count (slide p. 6: mass per tire ~20-30t). For your design, ~24.9t/wheel is reasonable (A380 ~28.75t/wheel).

- Load Distribution/Share:

  - Calculation: Main 90%, nose 10%.

  - Classification Justification: Class II (slide p. 7: empirical 85-95% main for steering/braking; Raymer p. 340: 85-95% main for tricycle stability).

  - Engineering Justification: Balances steering (nose >5%) and braking (main >85%); your 90/10 is midpoint, justifiable for widebody (handbook p. 20: collaborate with Stability for COM).

- Longitudinal Placement (Tip-Back Angle):

  - Calculation: β = tan^{-1}((x_cg - x_mg)/h_cg); main behind CG; gear length 3.92 m = Hw + 0.3.

  - Classification Justification: Class II (slide p. 7: semi-empirical constraints β > tail-strike, <25°; Raymer p. 342: geometry-based angle calcs).

  - Engineering Justification: Prevents tipping (slide p. 7: CG fwd of main); your formula matches (Raymer Fig. 11.5); h_cg=3.15 m reasonable (spec p. 5: airport compat.). "Main 5% away from engines" avoids interference (Raymer p. 362).

- Tail-Strike Angle:

  - Calculation: θ = tan^{-1}(7.05/6.07); tail height/body radius/tail factor/operational rotation.

  - Classification Justification: Class II (slide p. 8: geometry-based <15°, aim 8-9°; Raymer p. 342: arctan formula with gear height/dist to tail).

  - Engineering Justification: Protects fuselage (slide p. 8: rotation limit); your numbers imply l_tail ~6.07 m, h_tail ~7.05 m (including gear 3.92 m, radius 3.15 m), yielding ~49° (but target 12°—recheck assumptions). Raymer recommends 10-15° for jets.

Class II.5 Models (Physics-Based Calibrated – MDO Refinement)

These use detailed physics (e.g., energy absorption, materials) calibrated to data (handbook p. 3: "bridging preliminary/detailed"; slide p. 18: stroke λ, piston dia., material for mass).

- Shock Absorber Stroke (Not Fully Calculated in Doc):

  - Mentioned: Landing energy eq. (slide p. 15: 1/2 m V_z^2 = λ m g (0.75 x_s + 0.5 x_t); V_z=10 ft/s=3.048 m/s, x_t=6 in=0.152 m).

  - Classification Justification: Class II.5 (slide p. 15: physics-based with λ trade; Raymer p. 355: energy absorption calibrated to efficiencies).

  - Engineering Justification: Absorbs kinetic energy (slide p. 14); λ=1.1-1.3 for large civil (your doc implies collaboration with structures for "landing load case"). For MLW=448t, λ=1.2: x_s ≈0.42 m (as previous iterations). Calibrate to B777F data (spec p. 12).

- Physics-Based Mass (Not Fully Calculated):

  - Mentioned: Main fitting length = x_s +10%; slider dia. from 1,500 psi static; wall thickness for buckling/compression.

  - Classification Justification: Class II.5 (slide p. 18: refine Raymer with stroke/dia./material; Raymer p. 355: oleo sizing physics).

  - Engineering Justification: Calibrated to empirical (slide p. 18: Raymer + structural tube model); for your design, use steel (σ_y=1000 MPa, E=200 GPa); per leg force ~1,099 kN (90% MLW/4 legs), d~0.34 m, t~0.002 m → mass/leg ~7 kg (tube only, add 10-20% fittings). Total ~19,700 kg calibrated to Raymer.